Table 1. List of horizontally polarized antennas included in the resonant 60-meter group, with model identifications, dimensions referenced to outline figures, resonant impedance, free-space gain, gain differential from a free-space dipole, and calculated allowable power.  All antennas use AWG #14 copper wire.

Antenna			Model file name		Dimensions 			Resonant	Free-Space	Delta Gain	Allowable

Feet		Meters		Impedance	Gain (dBi)	(dB)		Power (W)

(Ohms)

1/2-wavelength dipole		dpl60-fs			L = 89.2		L=27.19		73.7		2.04		-----		50.0

1-wavelength vertically 		q60-fs-bf		L=47.9		L=14.6		127.0		3.14		1.10		38.8

oriented quad loop, bottom-fed

1-wavelength equilateral 		eqd60-fs-bf		L=64.32		L=55.7		117.4		2.80		0.76		42.0

delta loop, bottom-fed 					H=55.7		H=16.98

1-wavelength right-angle		rad60-fs-bf 		LB=79.0	LB=24.08	196.4		2.42		0.38		45.8

delta loop, bottom-fed 					LA=55.86	LA=17.03

H=39.5		H=12.04

1-wavelength rectangle, 		rect60-fs-bf		L=72.5		L=22.1		259.3		2.25		0.21		47.6

bottom-fed						H=20.0		H=6.1

2-element Moxon rectangle	mox60-fs		A=66.99	A=20.42	56.2		5.72		3.68		21.4

B=10.36	B=3.16

C=1.48		C=0.45

D=12.32	D=3.75

E=24.16	E=7.36

2-element driver-reflector Yagi	2lyag60-fs		LR=91.7	LR=27.95	42.0		6.07		4.03		19.8

LDR=87.64	LDR=26.71

SP=26.6	SP=8.11

3-element Yagi			3lyag60-fs		LR=91.2	LR=27.8	27.1		7.63		5.59		13.8

LDR=89.0	LDR=27.13

SP1=28.6	SP1=8.72

LDI=86.0	LDI=26.21

SP2=32.4	SP2=9.87

Table 2. List of antennas included in the horizontally polarized multi-band group, with model identifications, dimensions referenced to outline figures, free-space gain, gain differential from a free-space dipole, and calculated allowable power.  All antennas use AWG #14 copper wire.  Free-space dipole gain = 2.04 dBi.

Antenna			Model file name		Dimensions 			Impedance	Free-Space	Delta Gain	Allowable

Feet		Meters		(Ohms)		Gain (dBi)	(dB)		Power (W)

135' doublet			dblt135-60-fs		L=135.0		L=41.15		400 + j1128	2.67		0.63		43.2

102' doublet			dblt102-60-fs		L=102.0		L=31.09		113 + j 250	2.18		0.14		48.4

67' doublet			dblt67-60-fs		L=67.0		L=20.42		34.8 - j 439	1.83		-0.21		52.5

extended double Zepp		edz60-fs		L=229.0		L=69.8		176 - j 986	4.96		2.92		25.5

8JK				8jk60-fs			L=183.2		L=55.84		20.3 - j 250	6.92		4.88		16.3

W=45.8		W=13.96

lazy-H				lh60-fs			L=183.2		L=55.84		24.5 + j 1.8	8.00		5.96		12.7

H=91.6		H=27.92

80-m 2-wavelength loop		hohpl80-60-fs-cf		L=140.0		L=42.67		252 - j 27	4.97		2.93		25.5

square, corner-fed					C=560.0	C=170.69

80-m 2-wavelength loop		hohpl80-60-fs-sf		L=140.0		L=42.67		248 - j 39	3.04		1.00		39.7

square, side-fed					C=560.0	C=170.69

80-m 2-wavelength loop		hohpl80-tri-60-fs-cf	L=186.0		L=56.69		112 + j 19	3.05		1.01		39.6

triangle, corner-fed					C=560.0	C=170.69

80-m 2-wavelength loop		hohpl80-tri-60-fs-sf	L=186.0		L=56.69		130 + j 6	2.91		0.87		40.9

triangle, side-fed					C=560.0	C=170.69

60-m 2-wavelength loop		hohpl60-fs-cf		L=90.0		L=27.43		79 - j 334	1.18		-0.86		60.9

square, corner-fed					C=360.0	C=109.73

60-m 2-wavelength loop		hohpl60-fs-sf		L=90.0		L=27.43		241 - j 216	2.99		0.95		40.2

square, side-fed					C=360.0	C=109.73

60-m 2-wavelength loop		hohpl60-tri-fs-cf		L=120.0		L=36.58		256 - j 222	2.30		0.26		47.1

triangle, corner-fed					C=360.0	C=109.73

60-m 2-wavelength loop		hohpl60-tri-fs-sf		L=120.0		L=36.58		195 - j 315	2.59		0.55		44.1

triangle, side-fed					C=360.0	C=109.73

Table 3. List of antennas included in the vertically polarized, above ground, resonant 60-meter group, with model identifications, dimensions referenced to outline figures, resonant impedance, gain differential from a vertical dipole, and calculated allowable power.  All antennas use AWG #14 copper wire with the lowest wire point 5' above average ground.

Antenna			Model file name		Dimensions 			Resonant	TO Angle	Delta Gain	Allowable

Feet		Meters		Impedance	(degrees)	(dB)		Power (W)

(Ohms)

1/2-wavelength vertical		vdpl60-5		L=89.3		L=27.22		92.0		17		0.00		50.0

dipole

1-wavelength vertically 		q60-5-sf			L=47.0		L=14.33		217.7		22		0.92		40.5

oriented quad loop, side-fed

1-wavelength equilateral 		eqd60-5-sf		L=63.2		L=19.26		196.8		23		0.78		41.8

delta loop, side-fed, apex up				H=54.73	H=16.68

1-wavelength equilateral 		eqd60-5-ad-sf		L=64.2		L=19.57		175.5		19		0.68		42.8

delta loop, side-fed, apex down				H=54.73	H=16.68

1-wavelength right-angle 	rad60-5-sf		LB=79.26	LB=24.16	99.4		25		1.03		39.4

delta loop, side-fed, apex up				LA=55.99	LA=17.07

H=39.6		H=12.07

1-wavelength right-angle 	rad60-5-ad-sf		LB=80.0	LB=24.38	90.4		22		1.00		39.7

delta loop, side-fed, apex down				LA=56.56	LA=17.24

H=40.0		H=12.19

1-wavelength rectangle, 		rect60-5-sf		L=72.4		L=22.07		55.1		26		1.42		36.1

side-fed						H=20.75	H=6.32

Half-square			hs60-5			L=83.0		H=25.3		78.1		20		3.41		22.8

H=51.55	H=15.71

Bobtail curtain			bc60-5			L=166.2		L=50.66		84.5		20		4.91		16.1

H=50.05	H=15.26

Table 4. List of antennas included in the 60-meter vertical monopole group, with model identifications, dimensions referenced to outline figures, resonant impedance, gain, gain differential from a vertical dipole, and calculated allowable power.  All antennas use AWG #14 copper wire.  All radials are 0.001-wavelength below ground surface (2.2" or 5.6 cm) and are 1/4-wavelength long (45.81' or 13.96 m).

Soil Qualities:		Label		Abbr.		Conductivity (S/m)	Permittivity

Very Good	VG		0.0303			20

Good		G		0.005			13

Poor		P		0.002			13

Very Poor	VP		0.001			5

Antenna		Model		Soil	Monopole Length (LM)		Gain	TO Angle	Impedance	Delta Gain	Allowable

File name	Quality	Feet		Meters		(dBi)	(degrees)	(Ohms)		(dB)		Power (W)

Reference Vertical Dipole

1/2-wavelength		vdpl60-5	VG	L=89.3		L=27.22		2.40	14		94.7 + j1.3	-----		50.0

vertical dipole				G					0.00	17		92.0 - j0.8	-----		50.0

P					0.21	18		90.4 - j0.4	-----		50.0

VP					-0.75	20		87.1 - j2.1	-----		50.0

4-Radial System

1/4-wavelength		vmp60-4b	VG	LM=43.58	LM=13.28	0.80	20		46.1 - j3.7	-1.60		72.3

vertical monopole			G					-2.47	26		64.0 - j0.3	-2.47		88.3

P					-3.19	27		70.2 - j6.4	-3.40		109.4

VP					-5.34	29		101 + j16	-4.59		143.9

16-Radial System

1/4-wavelength		vmp60-16b	VG	LM=44.1	LM=13.44	1.94	20		37.6 - j1.2	-0.46		55.6

vertical monopole			G					-0.23	26		40.8 + j0.2	-0.23		52.7

P					-0.53	27		42.8 + j0.4	-0.74		59.3

VP					-1.37	29		38.5 + j4.7	-0.62		57.7

64-Radial System

1/4-wavelength		vmp60-64b	VG	LM=44.4	LM=13.53	2.47	20		33.8 + j0.1	0.07		49.2

vertical monopole			G					0.73	26		32.4 - j0.5	0.73		42.3

P					0.71	27		31.2 - j0.3	0.50		44.6

VP					-0.44	29		28.9 - j2.3	0.31		46.6