Schematic diagram of the converted amplifier and list of additional required parts.

BR2 — 100 V, 1 A bridge rectifier,

C — 100 µF, 50 V electrolytic capacitor.

C10-C17 — 540 µF, 450 V electrolytic capacitors.

C101 — 50 pF 1 kV variable capacitor.

C103 — 100 pF, 4 kV variable capacitor.

C104 — 250 pF, 4 kV variable capacitor. Original SB-220 tune capacitor, C55.

D1, D2 — K2AW 8 kV 1 A high voltage diode stacks.3

RL1 — 12 V DPDT mini relay.

RL2, RL3 — Vacuum relays, Gigavac GH1-12Vdc HAM.

T3 — Smallest 12 V, 0.5 A transformer available — needs to fit under SB-220 chassis.

L101, L102 — 10 turns 22 gauge enameled copper wire air wound on 5/8 inch threaded rod, 1 inch long.

L103 — 3 turns 1/4 inch copper refrigerator tubing, air wound on a 1.25 inch form, 3 inches long.

Z1, Z2 see text and Figures 8 and 9.

SoftStart Kit (Harbach Electronics) www.harbachelectronics.com.