; IAMBIC KEYER FOR PIC16C84 (c)1898 E SKELTON EI9GQ

LIST P=16F84

__CONFIG 0x3FFB		;RC, NO WDT,  PWR-UP, NO CP.

RA	EQU	05H		;PORT A

RB	EQU	06H		;PORT B

COUNT1	EQU	0CH		;COUNT FOR DELAY LOOP

COUNT2	EQU	0DH		; "

FLAG	EQU	0EH		;USER FLAGS

DIT_FLG	EQU	00H		;DOT FLAG

DIT_SW	EQU	00H		;DOT SWITCH (ON PADDLE KEY)

DAH_SW	EQU	01H		;DASH SWITCH (ON PADDLE KEY)

KEY	EQU	00H		;KEYING OUTPUT

ORG	00H		;PROGRAM EXECUTION BEGINS FROM HERE

GOTO	START

DAH	MOVLW	0x1E		;MAIN DOT/DASH DELAY LOOP

GOTO	CONT

DIT	MOVLW	0x0A

CONT	MOVWF	COUNT1

LP2	MOVLW	0x6E

MOVWF	COUNT2

LP1	DECFSZ	COUNT2,1

GOTO	LP1

DECFSZ	COUNT1,1

GOTO	LP2

RETLW	0		;END OF DELAY

START	BSF	3,5		;SWITCH TO F REGISTER PAGE 1

MOVLW	0FFH		;PORT A ALL INPUTS

MOVWF	RA

MOVLW	00H		;PORT B ALL OUTPUTS

MOVWF	RB

BCF	3,5		;SWITCH TO F REGISTER PAGE 0

MOVLW	0

MOVWF	RB		;MAKE SURE TX IS NOT KEYED ON POWER UP

MOVWF	FLAG		;CLEAR DOT AND DASH FLAGS

NEXT	BTFSS	RA,DIT_SW	;IS THE DIT PADDLE PRESSED?

GOTO	DOT

BTFSS	RA,DAH_SW	;IS THE DAH PADDLE PRESSED

GOTO	DASH

GOTO	NEXT		;LOOP UNTIL NEXT PADDLE PRESS

DOT	BTFSS	RA,DAH_SW	;ARE BOTH PADDLES PRESSED

GOTO	IAMBIC		;YES

DOT2	BSF	RB,KEY		;KEY TRANSMITTER

CALL	DIT		;DELAY FOR 1 DOT LENGTH

BCF	RB,KEY		;UNKEY TRANSMITTER

CALL	DIT		;DELAY FOR 1 DOT LENGTH

BSF	FLAG,DIT_FLG	;LAST MORSE ELEMENT SENT WAS A DOT (FOR IAMBIC ACTION)

GOTO	NEXT		;BACK AND WAIT UNTIL NEXT PADDLE PRESS

DASH	BTFSS	RA,DIT_SW	;ARE BOTH PADDLES PRESSED

GOTO	IAMBIC		;YES

DASH2	BSF	RB,KEY		;KEY TRANSMITTER

CALL	DAH		;DELAY FOR 1 DASH LENGTH (3 DOT LENGTHS)

BCF	RB,KEY		;UNKEY TRANSMITTER

CALL	DIT		;DELAY FOR 1 DOT LENGTH

BCF	FLAG,DIT_FLG	;LAST MORSE ELEMENT SENT WAS A DASH (FOR IAMBIC ACTION)

GOTO	NEXT 		;BACK AND WAIT UNTIL NEXT PADDLE PRESS

IAMBIC	BTFSC	FLAG,DIT_FLG	;CHECK TO SEE IF LAST CHAR SENT WAS A DOT OR A DASH

GOTO	DASH2		;IT WAS A DOT, SEND A DASH

GOTO	DOT2		;IT WAS A DASH, SEND A DOT

END