XW-2(CAS-3) Satellites Frequency Allocation

Alan Kung, BA1DU

2015-08-01

- Antenna:

XW-2A through XW-2F are using the exact same type of antenna, they are quarter wavelength deployable monopole whip antennas and made of steel tape, Including a UHF uplink antenna and a VHF downlink antenna for each satellite.

- Frequency Allocation Diagram:

453.000        435.056       435.116        435.176        435.236        435.296        435.350

Uplink   XW-2A        XW-2B        XW-2C        XW-2D        XW-2E        XW-2F

Downlink

145.620       145.6875       145.7525       145.8175       145.8825       145.9375       146.000

Digital Telemetry         CW Beacon           Linear Transponder

- XW-2 series of satellites:

Uplink:

| Satellite | Frequency (MHz) | Bandwidth (kHz) | Application |
| --- | --- | --- | --- |
| XW-2A (CAS-3A) | 435.030 - 435.050 | 20 | Linear Transponder (Spectrum Inverting) |
| XW-2B (CAS-3B) | 435.090 - 435.110 |  |  |
| XW-2C (CAS-3C) | 435.150 - 435.170 |  |  |
| XW-2D (CAS-3D) | 435.210 - 435.230 |  |  |
| XW-2E (CAS-3E) | 435.270 - 435.290 |  |  |
| XW-2F (CAS-3F) | 435.330 - 435.350 |  |  |

Downlink:

| Satellite | Application | Frequency (MHz) | Bandwidth (kHz) | RF Power (dBm) | Modulation |
| --- | --- | --- | --- | --- | --- |
| XW-2A (CAS-3A) | Digital Telemetry | 145.640 | 30 | 20 | 9.6/19.2kbps, GMSK |
|  | CW Beacon | 145.660 | 0.1 | 17 | 22wpm, CW |
|  | Linear Transponder | 145.665 - 145.685 | 20 | 20 |  |
| XW-2B (CAS-3B) | Digital Telemetry | 145.705 | 30 | 20 | 9.6/19.2kbps, GMSK |
|  | CW Beacon | 145.725 | 0.1 | 17 | 22wpm, CW |
|  | Linear Transponder | 145.730 - 145.750 | 20 | 20 |  |
| XW-2C (CAS-3C) | Digital Telemetry | 145.770 | 30 | 20 | 9.6/19.2kbps, GMSK |
|  | CW Beacon | 145.790 | 0.1 | 17 | 22wpm, CW |
|  | Linear Transponder | 145.795 - 145.815 | 20 | 20 |  |
| XW-2D (CAS-3D) | Digital Telemetry | 145.835 | 30 | 20 | 9.6/19.2kbps, GMSK |
|  | CW Beacon | 145.855 | 0.1 | 17 | 22wpm, CW |
|  | Linear Transponder | 145.860 - 145.880 | 20 | 20 |  |
| XW-2E (CAS-3E) | Digital Telemetry | 145.890 | 16 | 20 | 9.6kbps, GMSK |
|  | CW Beacon | 145.910 | 0.1 | 17 | 22wpm, CW |
|  | Linear Transponder | 145.915 - 145.935 | 20 | 20 |  |
| XW-2F (CAS-3F) | Digital Telemetry | 145.955 | 16 | 20 | 9.6kbps, GMSK |
|  | CW Beacon | 145.975 | 0.1 | 17 | 22wpm, CW |
|  | Linear Transponder | 145.980 - 146.000 | 20 | 20 |  |

- CAS-3 series of other satellites:

| Satellite | Emission direction | Application | Frequency (MHz) | Bandwidth (kHz) | Modulation |
| --- | --- | --- | --- | --- | --- |
| DCBB (CAS-3G) | Downlink | Digital Telemetry | 145.475 | 16 | 9.6kbps, GMSK |
|  |  |  | 437.950 |  |  |
| LilacSat -2 (CAS-3H) | Uplink | APRS | 144.390 | 16 | AFSK |
|  |  | FM Transponder | 144.350 | 16 | FM |
|  | Downlink | APRS | 144.390 | 16 | AFSK |
|  |  | CW Beacon | 437.200 | 0.1 | CW |
|  |  | FM Transponder | 437.225 | 16 | FM |
| NUDT-Phone-Sat (CAS-3I) | Downlink | Digital Telemetry | 437.300 | 28 | 9.6kbps, FSK |